#include <absacc.h>

#include <reg52.h>

#define uchar unsigned char

#define uint unsigned int

#define DAC0832 P2

#define DATA 57   //定时器延时

#define num1 1

#define num2 2

#define num3 3

#define num4 4

sbit k1=P3^0;

sbit k2=P3^1;

sbit k3=P3^2;

sbit k4=P3^3;

uint key_num;

uchar flag=0;

uchar num=0;

uint code sin_table[] =        //正弦波输出表

{

0x80,0x83,0x86,0x89,0x8D,0x90,0x93,0x96,0x99,0x9C,0x9F,0xA2,0xA5,0xA8,0xAB,0xAE,

0xB1,0xB4,0xB7,0xBA,0xBC,0xBF,0xC2,0xC5,0xC7,0xCA,0xCC,0xCF,0xD1,0xD4,0xD6,0xD8,

0xDA,0xDD,0xDF,0xE1,0xE3,0xE5,0xE7,0xE9,0xEA,0xEC,0xEE,0xEF,0xF1,0xF2,0xF4,0xF5,

0xF6,0xF7,0xF8,0xF9,0xFA,0xFB,0xFC,0xFD,0xFD,0xFE,0xFF,0xFF,0xFF,0xFF,0xFF,0xFF,

0xFF,0xFF,0xFF,0xFF,0xFF,0xFF,0xFE,0xFD,0xFD,0xFC,0xFB,0xFA,0xF9,0xF8,0xF7,0xF6,

0xF5,0xF4,0xF2,0xF1,0xEF,0xEE,0xEC,0xEA,0xE9,0xE7,0xE5,0xE3,0xE1,0xDF,0xDD,0xDA,

0xD8,0xD6,0xD4,0xD1,0xCF,0xCC,0xCA,0xC7,0xC5,0xC2,0xBF,0xBC,0xBA,0xB7,0xB4,0xB1,

0xAE,0xAB,0xA8,0xA5,0xA2,0x9F,0x9C,0x99,0x96,0x93,0x90,0x8D,0x89,0x86,0x83,0x80,

0x80,0x7C,0x79,0x76,0x72,0x6F,0x6C,0x69,0x66,0x63,0x60,0x5D,0x5A,0x57,0x55,0x51,

0x4E,0x4C,0x48,0x45,0x43,0x40,0x3D,0x3A,0x38,0x35,0x33,0x30,0x2E,0x2B,0x29,0x27,

0x25,0x22,0x20,0x1E,0x1C,0x1A,0x18,0x16,0x15,0x13,0x11,0x10,0x0E,0x0D,0x0B,0x0A,

0x09,0x08,0x07,0x06,0x05,0x04,0x03,0x02,0x02,0x01,0x00,0x00,0x00,0x00,0x00,0x00,

0x00,0x00,0x00,0x00,0x00,0x00,0x01,0x02,0x02,0x03,0x04,0x05,0x06,0x07,0x08,0x09,

0x0A,0x0B,0x0D,0x0E,0x10,0x11,0x13,0x15,0x16,0x18,0x1A,0x1C,0x1E,0x20,0x22,0x25,

0x27,0x29,0x2B,0x2E,0x30,0x33,0x35,0x38,0x3A,0x3D,0x40,0x43,0x45,0x48,0x4C,0x4E,

0x51,0x55,0x57,0x5A,0x5D,0x60,0x63,0x66,0x69,0x6C,0x6F,0x72,0x76,0x79,0x7C,0x7E

};

void delay(uint a)   //延时函数

{

uint i,j;

for(i=0;i<a;i++)

for(j=0;j<120;j++);

}

uint Key_Scan()    //按键扫描

{

static uint key_up;

if((!k1||!k2||!k3||!k4)&&key_up)

{

key_up=0;

delay(10);

if(k1==0)

{

return num1;

}

else	if(k2==0)

{

return num2;

}

else	if(k3==0)

{

return num3;

}

else	if(k4==0)

{

return num4;

}

}

else if(k1&&k2&&k3&&k4)  //无按键按下时

{

key_up=1;

}

return 0;                 //返回0

}

void Time0_Init()   //定时器初始化

{

TMOD=0x01;

TH0=(65536-DATA)/256;

TL0=(65536-DATA)%256;

ET0=1;

TR0=1;

EA=1;

}

void stair()   //锯齿波

{

uint i;

for(i=0;i<255;i++)

{

DAC0832=i;

TR0=1;

while(flag==0);

flag=0;

}

}

void aquare()    //方波

{

uint i;

DAC0832=0xff;

for(i=0;i<125;i++)

{

TR0=1;

while(flag==0);

flag=0;

}

DAC0832=0x00;

for(i=0;i<125;i++)

{

TR0=1;

while(flag==0);

flag=0;

}

}

void trian()   //三角波

{

uint i;

for(i=0;i<125;i++)

{

DAC0832=i;

TR0=1;

while(flag==0);

flag=0;

}

for(i=125;i>0;i--)

{

DAC0832=i;

TR0=1;

while(flag==0);

flag=0;

}

}

void sin()   //正弦

{

uint i;

for(i=0;i<255;i++)

{

DAC0832=sin_table[i];

TR0=1;

while(flag==0);

flag=0;

}

}

void main()

{

P2=0xff;

Time0_Init();

while(1)

{

key_num=Key_Scan();

if(key_num==1)

{

num=1;

}

else if(key_num==2)

{

num=2;

}

else if(key_num==3)

{

num=3;

}

else if(key_num==4)

{

num=4;

}

switch(num)

{

case 1:  aquare();break;  //方波

case 2:  trian(); break;  //三角波

case 3:  stair();break;   //锯齿波

case 4:  sin();  break;   //正弦

}

}

}

void zd() interrupt 1

{

TH0=(65536-DATA)/256;        //重装载

TL0=(65536-DATA)%256;

TR0=0;

flag=1;

}